# NUON Software Authentication

Revision 2	March 8, 2000

This document describes the techniques used to authenticate any software loaded on to the NUON platform. The purpose of this authentication is to ensure that VM Labs receives appropriate royalty payments for NUON applications.

The techniques used to perform authentication are described here, along with some of the thinking behind NUON authentication.

This authentication discussion should be considered quite confidential, and the details here discussed with outside parties only where necessary, such as for disk mastering. There is little here that could not be worked out by reverse engineering, but we do consider it to be worth obfuscating this process as much as possible.

The fundamental part of the authentication is based on a public / private key encryption scheme. This implies that the private keys are one of VM Labs’ crown jewels, and must be guarded appropriately.

## Application Types

NUON applications basically all lok for authentication purposes, described as hybrid and game, although these classes both include other types than their names suggest. For the purpose of this discussion, downloaded drivers are also considered to be applications.

### Hybrid Applications

The name of this application type comes from their use on hybrid DVDs. These are DVD or CD disks which contain normal DVD video, DVD audio or CD data and NUON specific data. In a NUON player these disks have additional features that are not available in a regular player.

The defining characteristic of a hybrid application for this discussion is that it is not feasible to enable NUON scrambling, because the regular content on the disk has to remain accessible during or after running the hybrid application. Key features of disk based hybrid applications are:

- VM Labs should trust the source of the application code. As NUON scrambling is not enabled, “Trojan Horse” and other attacks are more feasible.

- The application must load as a single file. It is not permitted in any circumstances for it to read more executable code off the disk. This ensures that it can be fully authenticated.

There can only be a few hybrid applications that must be compatible with future generations of NUON hardware, such as Scorpio. The reason is that they are unlikely to be fully compatible, so compatible versions will have to be stored in the ROM of Scorpio based machines.

Hybrid applications are authenticated using the technique describe below under “General Full Authentication Technique”.

Hybrid applications may be loaded off any media, including CD. The type of media has no bearing on the authentication used for these.

Down-loaded drivers, from disk, the NUON serial device bus, or wherever, are fully authenticated in the same way as hybrid application.

### NUON Game Applications

Once again, this is not limited to games. Any application that is loaded from disk should be treated as a NUON game application, unless there is a damn good reason for treating it as a hybrid application.

The defining characteristic here is that NUON scrambling is enabled. However, because these applications are allowed to load executable code off the disk, we also check that the disk has not changed since VM Labs authenticated it. It is not feasible to check the whole disk, as it would take too long, so we randomly check sectors against a list of authenticated MD5 digests of their contents. This is discussed in more detail below.

Another firm requirement is that the scrambling key for a game should be changed from time to time in manufacturing. This makes it difficult to predict the key used by a title, which protects against some form of disk swap attacks, and other forms of piracy.

The authentication requirements for a NUON game are:

- Fully authenticate the boot file of the game.

- Randomly check disk sectors against an authenticated list of digests to ensure the disk is unchanged.

- Enable NUON scrambling before running the game.

## General Full Authentication Technique

This authentication technique is used to fully authenticate any NUON file or downloaded block. It is used whenever it is feasible to read and check all the data of an application or driver. When it is not practical to check all the data, this technique is still used on part of it. See below under the discussion of DVD Game Authentication.

A fully authenticated file is always signed using the El-Gamal public/private key encryption technique. This adds a 2048 byte digital signature to the file. This is the fundamental part of the authentication, and is what guarantees that the file has been approved by VM Labs.

The file to be authenticated has a header attached that contains some information about the file. This is described elsewhere.

## NUON Scrambling

NUON scrambling is a variation on the DVD CSS scrambling technique. It is descrambled in hardware within the CDI, which is the drive interface hardware in NUON. It may only be applied to DVD disks; it does not work on CDs.

The data is scrambled using a special key, which is written into the CDI and locked there before the game starts running. After this has occurred only scrambled sectors can be read, so the general DVD file system is no longer accessible. The only way to exit from this mode is to perform a hardware reset.

The purpose of scrambling is two-fold; it protects against some forms of disk swap attacks (see the discussion below), and it also makes game disks harder to copy, as part of the de-scrambling process involves reading header information.

The descrambling process requires that the correct key be written to the CDI. This key is extracted from the fully authenticated game boot file. Then for each sector, 6 bytes are read from the sector header and used to initialize the de-scrambling hardware. These bytes can be essentially random data.

The header structures are:

| Byte | DVD video sector | Description |
| --- | --- | --- |
| 0-3 | ID | 4 bytes containing sector information and sector number |
| 4-5 | IED | ID Error Detection code. This is a CRC of the ID. |
| 6-11 | CPR_MAI | Copyright management information. Used to specify DVD region and copy protection information. |

| Byte | NUON sector | Description |
| --- | --- | --- |
| 0-3 | ID | 4 bytes containing sector information and sector number |
| 4-9 | ND_INIT | NUON descrambling initialization. |
| 10-11 | Reserved | Unused. |

Note that the IED specified by the DVD video standard overlaps with the descrambler initialization data in the NUON sector header. Fortunately (as this was accidental), the descrambling initialization data can be anything, so the IED will still actually be present, but is also used for the ND_INIT. This does have implications for disk mastering, however, as the initialization data is locked to a particular sector ID, so you have to know the physical location of a NUON file to scramble it.

### NUON Game Mastering Flow

A NUON game consists of two files.

| BOOT.COF | This is the first executable file loaded and run when the game is started. It must fit entirely into the available memory. During mastering a header and authentication signature are added to the file, but these are not loaded as part of the game. |
| --- | --- |
| BOOT.DAT | This is the data available to be loaded by the game while it is running through appropriate API calls. This data is NUON scrambled, and consists of a single contiguous file. The game may choose any data format within this file that it likes. |

The game authors will present the two files above for mastering, along with any other material that is not part of the game, such as DVD movies, or whatever. Note that this other material is not available to the game while it is running because it will not be NUON scrambled.

The mastering flow is:

- Create a prototype header and dummy authentication signature for BOOT.COF so that it is the correct size. It will not yet contain the correct sector digests, scramble key or full authentication signature.

- Using an appropriate mastering tool, create an image of the disk.

- Scramble BOOT.DAT based on its known disk position, randomly choosing the 40-bit NUON scramble key.

- Substitute the scrambled BOOT.DAT into the disk image, including the modified headers.

- Generate MD-5 digests of all of the sectors (or blocks of sectors) on the disk. Substitute these digests into BOOT.COF.

- Using the VM Labs private key, sign BOOT.COF. Substitute this BOOT.COF into the disk image.

- Repeat from step 3 for the required number of scramble keys. Release the disk image for disk mastering.

Note that steps 1 and 2 could be performed by the software developer themselves. We could create special “no authentication” players that would work with these masters. This needs to be thought through carefully to make sure these players are not a risk themselves.

## BCA

An explanation from Bill Rehbock:

BCA stands for Burst Cutting Area, it is essentially a bar code (separate from the artwork bar code that can now be placed at the hub ring) just before the lead-in area of a DVD.  The BCA was used by DIVX to easily and cheaply uniquely serialize each disc.  The BCA of a disc get cut into the aluminum substrate after lamination and annealing.  The disc replicator gets the serial numbers to cut via a secure Internet connection from whoever is handing out the numbers (like VM Labs).  BCA can be from 12 to 188 bytes, so we could use say, 100 bytes for a common RSA key that would be used at boot-up for a run of discs and still offer the others for unique serialization to publishers.  The cost of putting BCA on a disc is only about 5 cents.

The Samsung and Toshiba drives do not support reading this, although the Thomson drive does. This means we cannot make it a standard feature for NUON, unfortunately. However where we can read it, it has two important benefits:

- We can check part of it against a copy stored in the signed authentication data. This should help us protect against casual disk copying, as we do not believe any disk writers can write to this area.

- We can use it to give each disk a unique ID. This has no current benefit for authentication, but does open up various business opportunities.

The ability to read the BCA should be mandatory (where possible) for all future DVD OEMs.

## Multiple Disk Masters

More from Bill Rehbock:

Another useful tid-bit of information... glass stampers get destroyed after pressing about 20,000 discs because they become unreliable, so if we want to change stuff in the data area also every 20,000 discs, that's ok, too, it doesn't really increase replication costs, as a new stamper gets made anyway.  If we change both keys, it should make it really hard to track and break.

This is useful as it make it reasonable to use multiple scramble keys. We will just request they make a new stamper with a different scramble key each time.

# Discussion Document for Software AuthenticATION

This document is based on a meeting held in Jan 1998, regarding Software Authenticity strategies for NUON.

There are two parts to this document, an initial summary by John Mathieson (at the end of the document, and quoted in italics in places), and a much fuller discussion by Louis Cardillo. I (John) have also been through it to update it to today’s thinking (January 2000), which has not changed much.

This document expands on our original discussion and looks at many other possible security threats, and some of the defenses we might employ.  There is also a list of just the questions we need to answer to finalize security related changes in the NUON system.

## Fundamental Requirements for Software Authentication

Some overall decisions that are justified in this document are summarized here:

Games will be published on DVD media, not CD. Games will be subject to encryption/signature checks of the boot code and a random selection of other parts fo the games data. Games code and data will be scrambled, and the CDI locked to a game specific scramble key before the game is run.

A special class of applications, including hybrid DVD NUON programs, may be loaded from any media, but is subject to the special restrictions that all of the code and data must loaded at once, and must be securely authenticated as VM Labs approved.

We do not require or expect that drive drawer opening will reset the system.

## Overall Goal

Authenticity checking can be viewed as a process, which is started at system boot time, and is preserved all the way up to running applications. If this chain can be broken at any stage and code executed outside VM Labs' control, then security has been lost. Maintaining this security is clearly critical to the business model.

This is a clear statement of the overall goal, but it is actually not this black and white.  Even if we employ all the security techniques mentioned in John's original summary, there are still many ways that code can be "executed outside of VM Labs' control".  In the end, we may decide not to defend against all these possible attacks on our revenue stream, but we need to properly think through the tradeoffs.

For every potential security hole, there are various technical and legal barriers that can achieve different levels of protection.  The crux of this whole issue is choosing a reasonable balance between the overall effectiveness and the overall cost of the security measures we adopt.

## Security Threats

- Unlicensed Games on DVD/CD

- Trojan Horse Game Loading From DVD/CD (Disk Swapping)

- Trojan Horse Game Loading From Other NUON Data Ports

- Game Loading From A Network Connection

- Pirating Of Downloadable Drivers

- System ROM Replacements

- Peripheral Chip Copy Protection

- Bit For Bit Copies of DVD/CD disks

### Unlicensed Games on DVD/CD

Stopping unlicensed DVD/CD application disks from just popping in and working is our most obvious and essential security goal.  Part of our strategy is based on legal defenses, such as contracts with OEMs and Developers, and as much patent and copyright protection as we can get related to code and data that works with our architecture, instruction set/compression, data formats, dram data organization, video data paths, etc.  However, we also need technical defenses good enough that we usually don't need to fall back on legal negotiations.

A standard first line of defense is to use a public key algorithm combined with a good digest algorithm, such as RSA-MD5, to verify that the DVD/CD data was encrypted by VM Labs.  The RSA-MD5 algorithms themselves are relatively unbreakable, but the system implementation around these algorithms is almost always vulnerable in some way.  The challenge is to come up with a design that does not allow the RSA-MD5 Media Check to be circumvented -- or at least one that makes circumvention so onerous that we are willing to live with that level of insecurity.  For example, for NUON to have a practical boot time, only a random subset of the disk can be RSA-MD5 checked.  If this subset is not random enough, unlicensed disks might duplicate just the sections that are checked.  Even if there's enough randomness to reject a disk, say, half the time, it might not bother the Unlicensed Developer's customers to just try again a few times.

If the RSA-MD5 system implementation is good enough, we may not need another line of defense.  But we might screw up and accidentally leave an easy back door.  Or we may just like the idea of adding another layer of difficulty for the hacker.  In either case, NUON specific de-scrambling in the CDI is a relatively cheap addition.  This scheme would extend the normal DVD de-scrambling system so that we also de-scramble all NUON application data that passes through the CDI interface.  The de-scrambling hardware will need to use a default key value at boot, but then a disk-specific key could be retrieved from the disk and written to a write-once key register in the CDI.  Trying to keep the key value itself secret doesn't add much to our security, and would be difficult to do without adding hardware locks to Aries that prevent debug access during the boot process.

The security of CDI de-scrambling is not based on anything fundamental like encryption or digests, nor does it attempt to force an Unlicensed Developer to hardware hack his customer's console.  It is actually just a form of "security by making reverse engineering a hassle".  Despite the relatively low level of increased security, CDI de-scrambling may still be worthwhile in terms of bang for the buck -- since it might cost so little "buck" to extend the DVD CSS hardware to handle our own special CDI de-scrambling.

### Trojan Horse Game Loading From DVD/CD (Disk Swapping)

Suppose some Devious Developer releases a licensed game in which you can press a magic combination of keys at the right moment and get to a secret "Game Loader" screen.  The screen tells the user how to load an unlicensed game -- the most obvious approach being to swap in a new DVD or CD disk.  Actually, a clever enough Devious Developer could subvert a legitimate, non-devious application and turn it into an Unlicensed Game Loader.  For example, many games open to a screen where they wait for the user to select which game or level to jump to next.  The user could just be told to pop in an unlicensed disk and select option A or whatever.  This disk would have been designed knowing what sector the application will next download from and at what point it will jump to the newly downloaded code.

This scenario highlights how difficult it can be to do a secure implementation of RSA-MD5 without leaving ridiculously simple holes.  For example, how do we force the swapped disk to go through an RSA-MD5 check?  If we cannot do this, then our security reduces to just the reverse-engineer-able CDI De-scrambling, with RSA-MD5 being of no help here.  This implies that we need a Door Open Mechanism that forces swapped disks to go through RSA-MD5 checking.  If physically defeating this mechanism requires the user to open and modify their console/player, this would hopefully scare off a large percentage of consumers.

If we want the Door Open Mechanism to allow multi-disk games to keep executing during a disk swap, then we cannot simply tie the "door open" signal to the NUON chip reset.  Instead, we might need a MPE0-only reset signal which allows the other MPEs to continue executing while MPE0 does the RSA-MD5 check.  Of course, this isn't secure at all unless MPE0 and CDI have a "private" mode which blocks access from other MPEs during the RSA-MD5 check.  This is possible to do, but is non-trivial.  An even more difficult question is whether we can allow an application to have access to non-encrypted audio and photo disks, without opening yet another gaping hole in our security.

As above, we can also use CDI De-scrambling as a second line of defense.  This would require the Devious Developer to scramble the data on his unlicensed disk with the same key used for the Trojan Horse disk.  The idea of forcing titles to be released multiple times with different scrambling each time (if game publishers will put up with it) makes this defense a bit stronger because then the unlicensed disk will only play on top of those copies of the Game Loader disk that share its scramble key.

A Legal barrier against the development of a Game Loader might have our contract with Developers make them agree not to hide such a Game Loader in their applications, with all sorts of penalties if it happens.  Are we already doing this?  Of course, this only raises the bar a little, since a small company might go out of business long before the penalties make up for the revenue lost to unlicensed software.

January 2000 Summary – John

Our conclusion subsequent to this was that we could not require OEMs to modify their drives to provide a hardware reset line. We therefore will rely on mastering disks with multiple scramble keys to protect against this attack. As drawer open is generally under NUON software control, this should be reasonable.

### Trojan Horse Game Loading From Other NUON Data Ports

NUON systems may have a variety of data sources besides DVD/CD disks, any of which could be used by a Trojan Horse Game Loader, for example: system-bus expansion slots, ROM bus expansion slots, controller-ports, digital video/audio-in, and even analog video/audio-in (with a little cleverness).  To use most of these data ports, a Devious Developer would have to design and sell some Game Loader Hardware, but if the port connector is accessible on the outside of the NUON box, then the consumer would not have to hack into the console/player itself -- so more consumers might be willing to try it out.  The most obvious form of Game Loader Hardware would be a cartridge-like peripheral, but it could be almost anything, say a dirt-cheap interface into a PC, or maybe just a dirt-cheap CD-drive (the Horror! the Horror!).  Obviously, ports with slow data rates could only download "slow" or "small" games, but this still leaves a big market for a Devious Developer.  And some of the data ports are actually much higher performance than the normal DVD/CD path through CDI -- and using them wouldn't even require our CDI de-scrambling to be reverse engineered!

A possible combination Technical-Legal defense here could require Developers to sign additional scary contracts to get access to certain data ports, such as video-in.  These access rights could be RSA'd and recorded on the disk.  During NUON's RSA/MD5 check of the disk, it could look at these access bits to decide whether to "lock" or "unlock" access to the ports.  These locks would be trivial to implement in Aries hardware with write-once port-enable registers, so trivial that we might want to do them despite their "bang" being rather limited.  What they accomplish is really just to restrict the total number of developers that might be able to develop such a Game Loader.  Is this worthwhile?

### Game Loading From A Network Connection

A NUON system with a modem connection presents an especially difficult security problem (if we choose to care about it at all) since it inherently has a "game download path" built in.  Under an extreme interpretation, even a Web Browser which executes downloaded Java might be considered to be a "game loader", fortunately one that is inherently so slow as to qualify as its own security technique.  However, a Trojan Horse Game Loader that can open a modem connection could download any binary the user was patient enough to wait for, then jump to it's first line and start executing.  If we write all NUON Web Browsers, perhaps we could guarantee they don't have such back doors, but a Trojan Horse Game Loader could come off a DVD/CD as well.

A NUON system with a cable network connection has much the same issues, except they're worse because the download rate is so much higher.  As John wrote, we might be able to rely on the cable companies end-to-end control of their system to protect against non-licensed game downloads.  But this seems less likely if they support any kind of Internet connection for a Web Browser or Email or whatever.  In that case, I doubt they could do a very secure job of controlling what gets downloaded.

Beyond the idea of port locking bits that I mentioned above, the only reasonable response may be to forget about further technical defenses and just rely on legal protection.  Are there any other workable ideas?  To highlight how difficult a problem this is, suppose that we went so far as to build RSA-MD5 in hardware so that we could check all instructions as they are loaded into MPE IRAM to guarantee that they had been encrypted by VM Labs.  Even this level of defense would be vulnerable to revenue losses if someone developed a Trojan Horse Game "interpreter", admittedly not a worry for the highest performance games, but that still leaves a lot of territory undefended.

### Pirating Of Downloadable Drivers

As part of supporting various Peripherals, we need to create a Downloadable Driver mechanism.  A Devious Developer could try to use this as a pirate-ready way to get unlicensed code, say a Game Loader, into a NUON system.  In some ways, this is even more dangerous than the similar Game Loader threats described above, because it might be doable without help from anyone that has signed a contract with VM Labs, and without hardware modifications to the console/player itself.  One defense would be to RSA-MD5 check the Downloadable Driver before installing it.  Again, the real issue becomes how easy or hard it is to circumvent this check.  If we take this threat seriously, more thinking might come up with some cheap Aries hardware techniques to help fortify things -- e.g. lock bits on the download capability, or a "new device" mechanism similar to the "door open" ideas above.

January 2000 Summary – John

We do take this threat seriously, and require all downloadable drivers to be fully authenticated by RSA-MD5 or similar checks.

### System ROM Replacements

Our RSA-MD5 Check code would typically go into the motherboard ROM.  As John described, this leaves us vulnerable to a "ROM swap" attack by hardware hackers.  We could design Aries so that MPE0 has a built-in boot program which passes control to external ROM code only if some weird checksum-like Digest calculation matches.  I might even be able to make this boot process work out of MPE0's DTROM, so no IROM would be necessary.  To be at all secure, the MPE would have to block external access to it's memory space from boot until the checksum check is done -- then, the mode could be disabled by writing to a write-once "unlock" bit.  Like CDI De-scrambling, it is important to realize that this is "security by making reverse engineering a hassle".  It can be reverse engineered, and will be if the stakes are high enough.

Is a ROM swap attack worth defending against at all, even to the relatively modest degree afforded by a System ROM Digest?  The fact that it requires the user's console/player to be opened and hardware hacked would hopefully scare off most consumers anyway.  On the other hand, a System ROM Digest would give us some control over OEMs, so they couldn't release systems without a VM Labs approved System ROM.  But on yet another hand, if the OEMs are also our semiconductor licensees, then they could easily figure out the Digest algorithm from the ROM data in the Aries Netlist.  So again, is a System ROM Digest worthwhile?

January 2000 Summary – John

For flash ROM systems, we require a locked boot block that authenticates the remainder of the flash memory. In Gemini, we will authenticate that locked block from boot code within the chip.

### Peripheral Chip Copy Protection

Though not strictly an issue of protecting our software royalty stream, if we choose to get into the peripheral business, we might want to protect against clone knock-offs of our peripherals.  The most obvious protection is to make it impossible to attach a peripheral to our consoles without violating our patents.   Similarly, to gain copyright protection, we may wish to force peripherals to include copyrighted information such as downloaded code used by a device driver, or a logo displayed on the screen during boot.  My understanding is that it might still be legal for a clone manufacturer to copy this downloaded information if it is the only possible way to make a peripheral work.  So one strategy we used at 3DO (and I have also read of it being used elsewhere) was to RSA-MD5 the downloaded information, and consider it blessed if it passes this check.  The fact that a near infinite number of possible bit patterns can pass this RSA-MD5 check supposedly strengthens our legal case for copyright protection -- despite it being practically impossible for anyone besides VM Labs to come up with one of those bit patterns.  This always seemed a bit esoteric to me, but I don't understand the legal background enough to judge.  In any case, we might consider copyright protection unnecessary if our peripheral patents are strong enough on their own.

January 2000 Summary – John

We have patent protection on the joystick initialization protocol.

### Bit For Bit Copies of DVD/CD disks

I won't launch into the entirety of my usual harangue here... other than to emphasize that it IS possible, despite being non-intuitive to many people, to completely eliminate revenue losses from disk copying.  This can be done if the business model is changed such that consumers pay specifically for their console (or a small number of consoles) to have access to a disk's content.  This can be secure and asy-to-use if the console has a modem and a unique ID (or even just a unique caller ID, as John once pointed out).  One last comment:  consumers do pay for "intangible" products in many scenarios, for example, when buying pay-per-view cable movies -- they just have to see some value in it.

## Possible Security Defenses That Imply Aries Hardware Changes

The following list summarizes the questions we need to answer before finalizing security-related changes to Aries.  These questions assume the context of the previous section's discussion, and ignore the many software-only and legal-only security measures described above.

- CDI De-scrambling:  Do we want to use this technique, given it's rather limited addition to our security?  If so, we need to make up our own scrambling algorithm, as it would obviously be stupid to use the DVD CSS LFSR circuits directly.
Jan 2000 answer – this is exactly what we did.

- Door Open Mechanism:  Do we want to use this technique to force newly inserted disks through our RSA-MD5 Media Check?  If so, is it enough to connect the console's "door open" signal to the Aries reset pin?  Or should we add a more restricted MPE0-reset signal and a "private" MPE0 execution mode to allow multi-disk games to continue execution during a disk swap?
Jan 2000 answer – this was not considered possible.

- Port Lock Bits:  This should be trivial to implement, but is the small increase in security worthwhile?  We could take the position that we will add this logic only to those ports which are already being modified or bug-fixed in Aries.
Jan 2000 answer – the NUON CDI descrambling key may be locked, as may the debug port.

- System ROM Digest:  Is this means of "verifying" the System ROM worthwhile?  It implies adding an IROM or changing MPE-0 to execute from DTROM, as well as a "private" MPE0 execution mode that needs more thought.
Jan 2000 answer – still ”to be implemented” on-chip.

For items B, C, and D above, we could decide to leave the hardware changes out of Aries, and still have the option of adding them into future variations of Aries.  This would just mean that we have less security in a system built around Aries compared to one built around those future chips -- but normal applications could run on either system (at least so far as the hardware is concerned).  However, for item A above, disk compatibility demands that if we intend to add de-scrambling hardware into future chips, then we need it in Aries as well.

John's Email After Our Jan 16 1998 Meeting

## Software Authenticity for NUON

This document attempts to set up an authenticity strategy for the MMA.

Authenticity checking can be viewed as a process which is started at system boot time, and is preserved all the way up to running applications. If this chain can be broken at any stage and code executed outside VM Labs' control, then security has been lost. Maintaining this security is clearly critical to the business model.

### Authenticity of the boot ROM

Before the NUON device transfers control to the boot ROM, it might be desirable to ensure that the boot ROM is valid. A non approved ROM could come from an OEM or be substituted after manufacture.

Software embedded on chip can ensure that the boot code is approved by VM Labs by checking for some form of checksum. This software can be made invisible once the system has booted, and the checksum made long enough to ensure that brute force attempts to determine it are not practicable.

This step may not be necessary if we both trust our OEMs and do not care about preventing people from removing the original ROM and substituting a different one. It might be subverted anyway if we create a software developer's ROM which then passes into the wrong hands, see below.

### Initial Authenticity of an Application

The software in the boot ROM has to determine that an application has been approved by VM Labs before transferring control to it. This step may not be necessary for set-top boxes, where the delivery medium is controlled from end to end, but is clearly vital for DVD or CD delivered applications.

Authenticity is probably best controlled through the use of an encrypted area of the disk. A public/private key scheme, such as RSA, is ideal for this. This encrypted area holds information about the remainder of the media, such as MD5 message digest data. Some areas of the disk are chosen randomly, and their MD5 digests compared to the expected value.

This process has the problem that security and boot time have to be traded off against each other. Any area of the disk that is not checked might not contain approved code, but checking the entire disk of not feasible. However, see the next section.

### Run-Time Authenticity of an Application

Because all CD and DVD data is delivered through the CDI interface, it is possible to use the DVD de-scrambling process to check data at run time. During the playing of a movie, a Title Key is used to descramble the received data. This mechanism can clearly also be used for applications.

This is valuable as it protects us against disk swapping attacks. However, it has two requirements:

First, the title key must be invisible to and unchangeable by the application. This prevents third parties determining a key and using it for their own unauthorized material. The register containing the key must be both write only and write once, i.e. locked against change. This means the system must be reset before another application can be run (i.e. there must be a hard-wired front panel reset.)

Second, applications must be re-mastered with different keys on an occasional basis. This prevents purveyors of un-approved software from being able to say that their code will run on "top" of another approved application. The title key is vulnerable to reverse engineering, so it must not be predictable for a given environment.

### Originality of an Application

CD, and eventually DVD, media can be copied by either a commercial pirate or a home user. In the developed world, I assume that commercial piracy can be controlled through the courts, but home copying is another issue.

On CD media it is feasible, as was done with Jaguar CDs, to detect home copies because the data is not correctly aligned with the time stamp in the sub-codes. This is because CD copiers doe not make perfect bit copies at the lowest level, but generate the time stream themselves (I think). However, I am not sure this is the case for DVD. I assume that the Hollywood companies will fight to prevent home users having the capability to copy their disks.

In summary, do we need to do anything here?

### Developer Systems

At some point it will be desirable for us to create developer versions of production systems that can run un-encrypted media, and the boot ROM of these might be a security weak point if it got into the wrong hands. We should ensure (if we care about ROM swap attacks) that these boot ROMs require some additional piece of hardware, such as a development system link, or some sort of dongle, to be present.